# Motor Gasoline BS-IV (IS 2796:2008 Amd.1) — Grade MG 91: Specification Summary

Engineering memo — Hyperion-generated deliverable

Provenance: Generated by Hyperion from corpus source DOC-010 (MRPL Specification for Motor Gasoline BS-IV, MG 91). This is a Hyperion-generated deliverable, not an original MRPL-issued document.

## Summary

Grade MG 91 under IS 2796:2008 (Amd.1, March 2010) requires RON 91 min and MON 81 min, with tight control on sulphur (50 mg/kg max), lead (0.005 g/l max) and benzene (1.0 % vol max). Volatility is bounded by E70 10-45, E100 40-70, RVP 60 kPa and seasonal VLI limits (summer 750, winter 950 max). Typical analysis values in the source sit comfortably inside every limit (e.g. RON 91.3, sulphur 25 mg/kg, lead < 0.001 g/l).

## Engineering interpretation

The matrix below is a 1:1 transcription of the source specification table. No values were estimated or rounded beyond the source text. Oxygenates other than ethanol at 5 % vol max play no role in the typical analysis (reported NA).

## Specification matrix (MG 91)

| Sl. | Parameter | Requirement | Unit | Test Method | Typical |
| --- | --- | --- | --- | --- | --- |
| i | Colour | As decided by refiners & marketers | — | Visual | Orange |
| ii | Density at 15°C | 720-775 | kg/m3 | IS 1448, P:16 | 751 |
| iii | E70 (recovery up to 70°C) | 10-45 | % vol | IS 1448, P:18 | 25 |
| iv | E100 (recovery up to 100°C) | 40-70 | % vol | — | 55 |
| v | E150 (recovery up to 150°C) | 75 min | % vol | — | 89 |
| vi | Final Boiling Point | 210 max | °C | — | 180 |
| vii | Residue | 2 max | % vol | — | 0.5 |
| viii | Research Octane Number (RON) | 91 min | — | IS 1448, P:27 | 91.3 |
| ix | Motor Octane Number (MON) | 81 min | — | IS 1448, P:26 | 83.5 |
| x | Existent Gum | 40 max | g/m3 | IS 1448, P:147 | 18 |
| xi | Sulphur, total | 50 max | mg/kg | IS 1448, P:34 | 25 |
| xii | Lead Content (as Pb) | 0.005 max | g/l | IS 1448, P:80 | < 0.001 |
| xiii | Reid Vapour Pressure at 38°C | 60 | kPa | IS 1448, P:39 | 47 |
| xiv | Vapour Lock Index, Summer / Winter | 750 / 950 max | — | — | 620 / 650 |
| xv | Benzene | 1.0 max | % vol | ASTM D 3606 | 0.50 |
| xvi | Copper strip corrosion (3h at 50°C) | Not more than 1 | class | IS 1448, P:15 | Class 1 |
| xvii | Olefin Content | 21 max | % vol | P:23 | 1.3 |
| xviii | Oxidation Stability | 360 min | minutes | P:28 | Above 360 |
| xix | Aromatic Content | 35 max | % vol | P:23 | 33.5 |
| xx | Oxygen Content | 2.7 max | % mass | ISO 1601 | NA* |
| xxi | Ethanol (oxygenate) | 5 max | % vol | ASTM D 4815 | — |

## Source attribution

Source: corpus DOC-010 (MRPL Specification for Motor Gasoline BS-IV, MG 91, 2 pages). Classification: Hyperion-generated from corpus source; not an original MRPL-issued document.